# Electromagnetic Compatibility Requirements (Based on DIN EN IEC 55025)

Chapter 1: General Requirements

## 1.1 Types of Interference Sources (as defined in the test plan)

Electromagnetic interference sources can be divided into two main types:

- Narrowband sources, such as vehicle electronic modules with clock generators, oscillators, digital logic circuits (e.g., microprocessors), and display units.

- Broadband sources, such as electric motors and ignition systems.

Note 1: Most vehicles or electrical/electronic components are sources of both narrowband and broadband interference, though some may emit only one type.

Note 2: Broadband sources can be classified into:

- Short-duration broadband interferers (e.g., washer pumps, side mirrors, electric windows)

- Long-duration broadband interferers (e.g., windshield washer motors, heating blowers, engine cooling)

For the application of this document, classification into types of interference is only used to simplify testing efforts. For example, the number of detectors may be reduced (e.g., omission of the average detector if the device is known to be a broadband source like a brushed DC motor). However, this document mandates that sources comply with limits for both types of detectors regardless of the type of interference.

## 1.2 Test Plan

A test plan must be drawn up for each item to be tested. This plan must specify:

- The frequency range to be tested

- Emission limit values

- Types and locations of antennas

- Requirements for the test report

- Supply voltage and other relevant parameters, including operating conditions of the device under test (DUT), as described in section 4.1.4

The test plan shall specify for each frequency range whether compliance can be achieved with mean and peak limits or with mean and quasi-peak limits.

### 4.1.4 Operating Conditions

Different operating conditions of the device under test (DUT) can influence the measurement results of interference emissions.

When testing components or modules, the DUT must operate under typical load and conditions as encountered in vehicles, ensuring maximum emission states are achieved. These conditions must be defined in the test plan.

To ensure proper operation during testing, a peripheral interface unit replicating the vehicle installation must be used. All essential sensor and actuator lines of the DUT must be connected to this unit. The unit must be capable of controlling the DUT as specified.

The peripheral interface unit may be placed inside or outside the shielded chamber. If placed inside, its emission levels must be at least 6 dB below the specified limits in the test plan.

Chapter 2: Measurement of General Interference Emission

For acceptable radio reception in vehicles using typical radio receivers, the interference voltage at the end of the antenna cable should not exceed the values specified in Table 1 and Table 2. If other types of receivers are used or different coupling models for interference propagation apply, the limits can be adjusted and specified individually by the vehicle manufacturer.

## Table 1 – Example for Emission Limits – Entire Vehicle – General

Table 1.1 Analog Broadcasting Services

| Service/Band | Frequency [MHz] | Peak Limit dB(µV) | Quasi-Peak Limit dB(µV) | Mean Limit dB(µV) | RBW |
| --- | --- | --- | --- | --- | --- |
| LW | 0,15 - 0,3 | 26 | 13 | 6 | 9 kHz |
| MW | 0,53 - 1,8 | 20 | 7 | 0 | 9 kHz |
| SW | 5,9 - 6,2 | 20 | 7 | 0 | 9 kHz |
| FM | 76 - 108 | 26 | 13 | 0 | 120 kHz |
| TV Band I | 41 - 88 | 16 | – | 6 | 120 kHz |
| TV Band III | 174 - 230 | 16 | – | 6 | 120 kHz |
| TV Band IV | 470 - 944 | 16 | – | 6 | 120 kHz |

Table 1.2 Digital Broadcasting Services

| Service/Band | Frequency [MHz] | Peak Limit dB(µV) | Quasi-Peak Limit dB(µV) | Mean Limit dB(µV) | RBW |
| --- | --- | --- | --- | --- | --- |
| DAB III | 171 - 245 | 26 | – | 16 | 1 MHz |
| TV Band III | 174 - 230 | 26 | – | 16 | 1 MHz |
| DTTB | 470 - 770 | 32 | – | 22 | 1 MHz |
| DAB-L-Band | 1447 - 1494 | 32 | – | 22 | 1 MHz |
| SDARS | 2320 - 2345 | 32 | – | 22 | 1 MHz |

Table 4.3 Mobile Services

| Service/Band | Frequency [MHz] | Peak Limit dB(µV) | Quasi-Peak Limit dB(µV) | Mean Limit dB(µV) | RBW |
| --- | --- | --- | --- | --- | --- |
| CB | 26 - 28 | 20 | 7 | 0 | 9 kHz |
| VHF | 30 - 54 | 20 | 7 | 0 | 120 kHz |
| VHF | 68 - 87 | 20 | 7 | 0 | 120 kHz |
| VHF | 142 - 175 | 20 | 7 | 0 | 120 kHz |
| Analog UHF | 380 - 512 | 20 | 7 | 0 | 120 kHz |
| RKE & TPMS 1 | 300 - 330 | 20 | – | 6 | 120 kHz |
| RKE & TPMS 2 | 420 - 450 | 20 | – | 6 | 120 kHz |
| Analog UHF | 820 - 960 | 20 | 7 | 0 | 120 kHz |
| GPS L5 | 1156,45 - 1196,45 | – | – | 10 | 9 kHz |
| BDS, B1l | 1553,098 - 1569,098 | – | – | -4,5 | 9 kHz |
| GPS L1 | 1567,42 - 1583,42 | – | – | 0 | 9 kHz |
| GLONASS L1 | 1590,781 - 1616,594 | – | – | 0 | 9 kHz |
| Wi-Fi/Bluetooth | 2402 - 2494 | 26 | – | 6 | 1 MHz |
| Wi-Fi | 5150 - 5350 | 26 | – | 6 | 1 MHz |
| Wi-Fi | 5470 - 5725 | 26 | – | 6 | 1 MHz |
| V2X (Wi-Fi) | 5 850 - 5 925 | 50 | – | 30 | 1 MHz |

## Table 2 – Example Emission Limits – Entire Vehicle – Digital Mobile Phones

| Service/Band | Frequenz [MHz] | Peak Limit dB(µV) | Mean Limit dB(µV) | RBW |
| --- | --- | --- | --- | --- |
| 4G | 460 - 467,5 | 32 | 12 | 1 MHz |
| 5G n71 (617 MHz - 652 MHz) | 617 - 652 | 32 | 12 | 1 MHz |
| 4G (617 MHz - 652 MHz) | 617 - 652 | 32 | 12 | 1 MHz |
| 5G n12 (729 MHz - 746 MHz) | 703 - 803 | 32 | 12 | 1 MHz |
| 5G n14 (758 MHz - 768 MHz) | 703 - 803 | 32 | 12 | 1 MHz |
| 5G n28 (758 MHz - 803 MHz) | 703 - 803 | - | - | 1 MHz |
| 5G n29 (717 MHz - 728 MHz) | 703 - 803 | - | - | 1 MHz |
| 4G (703 MHz - 803 MHz) | 703 - 803 | - | - | 1 MHz |
| 3G (729 MHz - 756 MHz) | 703 - 803 | 26 | 6 | 1 MHz |
| 3G (758 MHz - 768 MHz) | 703 - 803 | 26 | 6 | 1 MHz |
| 5G n20 (791 MHz to 821 MHz) | 791 - 821 | 32 | 12 | 1 MHz |
| 3G (791 MHz - 821 MHz) | 791 - 821 | 26 | 6 |  |
| 5G n5 (869 MHz - 894 MHz) | 852 - 894 | 32 | 12 | 1 kHz |
| 5G n18 (860 MHz - 875 MHz) | 852 - 894 | 32 | 12 | 1 kHz |
| 5G n26 (859 MHz - 894 MHz) | 852 - 894 | 32 | 12 | 1 kHz |
| 4G (852 MHz - 894 MHz) | 852 - 894 | 32 | 12 | 1 kHz |
| 3G (859 MHz - 894 MHz) | 852 - 894 | 26 | 6 | 1 kHz |
| 2G (869 MHz - 894 MHz) | 852 - 894 | 26 | 6 | 120 kHz |
| 5G n8 (925 MHz - 960 MHz) | 925 - 960 | 32 | 12 | 1 MHz |
| 4G (925 MHz - 960 MHz) | 925 - 960 | 32 | 12 | 1 MHz |
| 5G n8 (925 MHz - 960 MHz) | 925 - 960 | 32 | 12 | 1 MHz |
| 4G (925 MHz - 960 MHz) | 925 - 960 | 32 | 12 | 1 MHz |
| 3G (925 MHz - 960 MHz) | 925 - 960 | 26 | 6 | 1 MHz |
| 2G (925 MHz - 960 MHz) | 925 - 960 | 26 | 6 | 120 kHz |
| 5G n50 (1432 MHz - 1517 MHz) | 1 427 - 1 518 | 32 | 12 | 1 MHz |
| 5G n51 (1427 MHz - 1432 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n74 (1475 MHz - 1518 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n75 (1432 MHz - 1517 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n76 (1427 MHz - 1432 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n91 (1427 MHz - 1432 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n92 (1432 MHz - 1517 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n93 (1427 MHz - 1432 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 5G n94 (1432 MHz - 1517 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 4G (1427 MHz - 1518 MHz) | 1427 - 1518 | 32 | 12 | 1 MHz |
| 3G (1 452 MHz - 1 496 MHz) | 1 427 - 1 518 | 26 | 6 | 1 MHz |
| 4G | 1 525 - 1 559 | 32 | 12 | 1 MHz |
| 5G n3 (1805 MHz - 1880 MHz) | 1805 - 1880 | 32 | 12 | 1 MHz |
| 4G (1805 MHz - 1880 MHz) | 1805 - 1880 | 32 | 12 | 1 MHz |
| 3G (1 805 MHz - 1 880 MHz) | 1805 - 1880 | 26 | 6 |  |
| 2G (1 805 MHz - 1 880 MHz) | 1805 - 1880 | 26 | 6 | 120 kHz |
| 5G n2 (1930 MHz - 1990 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 5G n2 (1930 MHz - 1990 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 5G n25 (1930 MHz - 1995 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 5G n34 (2010 MHz - 2025 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 5G n39 (1880 MHz - 1920 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 5G n70 (1995 MHz - 2020 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 4G (1850 MHz - 2025 MHz) | 1850 - 2025 | 32 | 12 | 1 MHz |
| 3G (1850 MHz - 1995 MHz) | 1850 - 2025 | 26 | 6 | 1 MHz |
| 3G (2010 MHz - 2025 MHz) | 1850 - 2025 | 26 | 6 | 1 MHz |
| 2G (1930 MHz - 1990 MHz) | 1850 - 2025 | 26 | 6 | 120 kHz |
| 5G n1 (2110 MHz - 2170 MHz) | 2110 - 2200 | 32 | 12 | 1 MHz |
| 5G n65 (2110 MHz - 2200 MHz) | 2110 - 2200 | 32 | 12 | 1 MHz |
| 5G n66 (2110 MHz - 2200 MHz) | 2110 - 2200 | 32 | 12 | 1 MHz |
| 4G (2110 MHz - 2200 MHz) | 2110 - 2200 | 32 | 12 | 1 MHz |
| 3G (2110 MHz - 2170 MHz) | 2110 - 2200 | 26 | 6 | 1 MHz |
| 5G n30 (2350 MHz - 2360 MHz) | 2300 - 2400 | 32 | 12 | 1 MHz |
| 5G n40 (2300 MHz - 2400 MHz) | 2300 - 2400 | 32 | 12 | 1 MHz |
| 4G (2300 MHz - 2400 MHz) | 2300 - 2400 | 32 | 12 | 1 MHz |
| 3G (2300 MHz - 2400 MHz) | 2300 - 2400 | 26 | 6 | 1 MHz |
| 5G n53 (2483,5 MHz - 2495 MHz) | 2483,5 - 2495 | 32 | 12 | 1 MHz |
| 5G n7 (2620 MHz - 2690 MHz) | 2496 - 2690 | 32 | 12 | 1 MHz |
| 5G n38 (2570 MHz - 2620 MHz) V2X | 2496 - 2690 | 32 | 12 | 1 MHz |
| 5G n41 (2496 MHz - 2690 MHz) | 2496 - 2690 | 32 | 12 | 1 MHz |
| 5G n90 (2496 MHz - 2690 MHz) | 2496 - 2690 | 32 | 12 | 1 MHz |
| 4G (2496 MHz - 2690 MHz) | 2496 - 2690 | 32 | 12 | 1 MHz |
| 3G (2570 MHz - 2690 MHz) | 2496 - 2690 | 26 | 6 | 1 MHz |
| 5G n48 (3550 MHz - 3700 MHz) | 3300 - 4200 | 32 | 12 | 1 MHz |
| 5G n77 (3300 MHz - 4200 MHz) | 3300 - 4200 | 32 | 12 | 1 MHz |
| 5G n78 (3300 MHz - 3800 MHz) | 3300 - 4200 | 32 | 12 | 1 MHz |
| 4G (3300 MHz - 3800 MHz) | 3300 - 4200 | 32 | 12 | 1 MHz |
| 3G (3510 MHz - 3590 MHz) | 3300 - 4200 | 26 | 6 | 1 MHz |
| 5G n79 (4400 MHz - 5000 MHz) | 4400 - 5000 | 32 | 12 | 1 MHz |
| 4G | 5150 - 5925 | 32 | 12 | 1 MHz |
| 5G n47 (5855 MHz - 5925 MHz) V2X | 5855 - 5925 | 32 | 12 | 1 MHz |
| V2X (4G) | 5855 - 5925 | 32 | 12 | 1 MHz |

## Table 3 – Spectrum Analyzer Parameters

Table 3.1 Analog Broadcast Services

| Service/Band | Frequency Range MHz | RBW (-3 dB) | Min. Sweep Time | RBW (-6 dB) | Min. Sweep Time | RBW (-3 dB) | Min. Sweep Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LW | 0,15 - 0,30 | 9 kHz/10 kHz | 10 s/MHz | 9 kHz/10 kHz | 200 s/MHz | 9 kHz/10 kHz | 10 s/MHz |
| MW | 0,53 - 1,8 | 9 kHz/10 kHz | 10 s/MHz | 9 kHz/10 kHz | 200 s/MHz | 9 kHz/10 kHz | 10 s/MHz |
| SW | 5,9 - 6,2 | 9 kHz/10 kHz | 10 s/MHz | 9 kHz/10 kHz | 200 s/MHz | 9 kHz/10 kHz | 10 s/MHz |
| FM | 76 - 108 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz | 100 ms/MHz |
| TV Band I | 41 - 88 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz | 100 ms/MHz |
| TV Band III | 174 - 230 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz | 100 ms/MHz |
| TV Band IV | 470 - 944 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz | 100 ms/MHz |

| Service/Band | Frequency Range MHz | RBW (-3 dB) | Min. Sweep Time | RBW (-6 dB) | Min. Sweep Time | RBW (-3 dB) | Min. Sweep Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAB III | 167 - 245 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| TV Band III | 174 - 230 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| DTTV | 470 - 770 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| DAB-L-Band | 1447 - 1494 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| SDARS | 2320 - 2345 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |

Table 3.2 Digital Broadcast Services

Table 3.2 Mobile Services

| Service/Band | Frequency Range MHz | RBW (-3 dB) | Min. Sweep Time | RBW (-6 dB) | Min. Sweep Time | RBW (-3 dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CB | 26 - 28 | 9 kHz/10 kHz | 10 s/MHz | 9 kHz/10 kHz | 200 s/MHz | 9 kHz/10 kHz |
| VHF | 30 - 54 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz |
| VHF | 68 - 87 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz |
| VHF | 142 - 175 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz |
| Analog UHF | 380 - 512 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz |
| RKE & TPMS 1 | 300 - 330 | 100 kHz/ 120 kHz | 100 ms/MHz | Not applicable | Not applicable | 100 kHz/ 120 kHz |
| RKE & TPMS 2 | 420 - 450 | 100 kHz/ 120 kHz | 100 ms/MHz | Not applicable | Not applicable | 100 kHz/ 120 kHz |
| Analog UHF | 820 - 960 | 100 kHz/ 120 kHz | 100 ms/MHz | 100 kHz/ 120 kHz | 20 s/MHz | 100 kHz/ 120 kHz |
| GPS L5 | 1156,45 - 1196,45 | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz/10 kHz |
| BDS, B1l | 1553,098 - 1569,089 | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz/10 kHz |
| GPS L1 | 1567,42 - 1583,42 | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz/10 kHz |
| GLONASS L1 | 1590,781 - 1616,594 | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz/10 kHz |
| Wi-Fi/Bluetooth | 2402 - 2494 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz |
| Wi-Fi | 5150 - 5350 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz |
| Wi-Fi | 5470 - 5725 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz |
| V2X (Wi-Fi) | 5850 - 5925 | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz |

Table 3.2 Digital Mobile Phone Services

| Service/Band | Frequency Range MHz | RBW (-3 dB) | Min. Sweep Time | RBW (-6 dB) | Min. Sweep Time | RBW (-3 dB) | Min. Sweep Time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 100 kHz/ 120 kHz | 100 ms/MHz | Not applicable |  | 100 kHz/ 120 kHz | 100 ms/MHz |
| 3G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| 4G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |
| 5G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 100 ms/MHz | Not applicable | Not applicable | 1 MHz | 100 ms/MHz |

## Table 4 – Characteristics of the Sweep Receiver

Table 4.1 Analog Broadcast Services

| Service/Band | Frequency Range (MHz) | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LW | 0,15 - 0,30 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |
| MW | 0,53 - 1,8 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |
| SW | 5,9 - 6,2 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |
| FM | 76 - 108 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| TV Band I | 41 - 88 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| TV Band III | 174 - 230 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| TV Band IV | 470 - 944 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |

Table 4.2 Digital Broadcast Services

| Service/Band | Frequency Range (MHz) | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAB III | 167 - 245 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| TV Band III | 174 - 230 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| DTTV | 470 - 770 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| DAB-L-Band | 1447 - 1494 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| SDARS | 2320 - 2345 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |

Table 4.3 Mobile Services

| Service/Band | Frequency Range (MHz) | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB | 26 - 28 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |
| VHF | 30 - 54 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| VHF | 68 - 87 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| VHF | 142 - 175 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| Analog UHF | 380 - 512 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| RKE & TPMS 1 | 300 - 330 | 120 kHz | 50 kHz | 5 ms | Not applicable | Not applicable | Not applicable | 120 kHz | 50 kHz | 5 ms |
| RKE & TPMS 2 | 420 - 450 | 120 kHz | 50 kHz | 5 ms | Not applicable | Not applicable | Not applicable | 120 kHz | 50 kHz | 5 ms |
| Analog UHF | 820 - 960 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |
| GPS L5 | 1156,45 - 1196,45 | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz | 5 kHz | 5 ms |
| BDS, B1l | 1553,098 - 1569,089 | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz | 5 kHz | 5 ms |
| GPS L1 | 1567,42 - 1583,42 | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz | 5 kHz | 5 ms |
| GLONASS L1 | 1590,781 - 1616,594 | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable | 9 kHz | 5 kHz | 5 ms |
| Wi-Fi/Bluetooth | 2402 - 2494 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| Wi-Fi | 5150 - 5350 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| Wi-Fi | 5470 - 5725 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| V2X (Wi-Fi) | 5850 - 5925 | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |

Table 4.4 Digital Mobile Services

| Service/Band | Frequency Range (MHz) | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time | RBW at –6 dB | Max. Step Size | Minimum Measurement Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 120 kHz | 50 kHz | 5 ms | Not applicable |  |  | 120 kHz | 50 kHz | 5 ms |
| 3G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| 4G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |
| 5G | The frequency bands associated with these technologies are specified in detail in the emission limit tables. | 1 MHz | 500 kHz | 50 ms | Not applicable | Not applicable | Not applicable | 1 MHz | 500 kHz | 50 ms |

Figure 8a – GPS-Band L5 1 156,45 MHz to 1 196,45 MHz

Figure 8b – BDS-, B1l-Band 1 553,098 MHz to 1 569,098 MHz

Figure 8c – GPS-Band 1 567,42 MHz to 1 583,42 MHz

Figure 8d – GLONASS-Band 1 590,781 MHz to 1 616,594 MHz

Figure 8 - Details of Mean Value Limits for GPS, BDS, B1l, and GLONASS Bands – Complete Vehicle

Note: If an active antenna is used, the noise floor may increase. The additional noise floor depends on the type of antenna and is subtracted from the measured value using the following formula to determine the actual value of the interference signal (all terms in µV).

A relaxation of the limit due to the noise floor of the active antenna does not ensure that the limit is complied with. Subsequent design modifications of the active antenna may result in non-compliance with the limit. This issue is under further investigation. Annex D describes a method for determining the noise floor of the active antenna.

The values specified in Table 5 must not be exceeded. If other types of receivers are used or different coupling models for interference propagation apply, the limits may be adjusted and specified individually by the vehicle manufacturer.

Annex D
(informative)

Guide for Determining the Noise Floor of Active Vehicle Antennas

To determine the noise floor of an active antenna installed in a vehicle, three steps are necessary:

- Measurement of the equipment noise floor (Uequipment_noise) of the measurement setup (receiver plus impedance adapter), using a coaxial cable termination at the RF input of the impedance adapter within the applicable frequency range of the active antenna (see Figure D.1 for measurement setup).

- Measurement of the combined noise (Uequipment_noise + antenna_noise) from the active vehicle antenna, including the equipment noise floor (see Figure D.2 for measurement setup).

- Calculation of the antenna noise floor using Equation (D.1) (all values in µV):

Figure D.1 – Example of Vehicle Measurement Setup for Measuring Equipment Noise

Legend

- Measuring instrument

- Anechoic chamber

- Feedthrough connector

- Resistor matching the impedance of the coaxial cable

- Vehicle antenna amplifier

- Antenna coaxial cable

- High-quality, double-shielded coaxial cable (50 Ω)

- Impedance matching unit

- Short connection to the vehicle's radio receiver housing

- Housing of the radio receiver in the vehicle

Figure D.2 – Example of Vehicle Measurement Setup for Measuring Antenna Noise

Legend

- Measuring instrument

- Anechoic chamber

- Feedthrough connector

- Vehicle antenna amplifier

- Antenna amplifier connector

- Antenna coaxial cable

- High-quality coaxial cable, e.g., double-shielded (50 Ω)

- External 12 V battery

- Impedance matching unit

- Modified coaxial T-connector or short connection to the vehicle’s radio receiver housing

- Housing of the radio receiver in the vehicle